Architectural Phase Specs

NOTES

Review Green room, Broadwater,

Weekly status update submitted electronically to MCH to include : current status of all active projects, projected status of all active projects

- Floor plans to include

  - Directional dimensions for all rooms, halls, closets, storage areas, etc

  - Room SF (chart?)

  - Floor conditioned AND unconditioned SF decking SF

  - LAR and FAR calculations chart

  - Showers and vanities Dimensions

  - Elevation drawings to show Base Flood, Design Flood, Max Ridge Height, and road relevant elevation (if necessary)

  - Cross sections with Floor to floor dimension labeled

  - Lot Survey

  - Largest possible bed graphically represented in each bedroom on the appropriate wall

  - Foundation dimensions (including offsets)

  - Flood vents labeled as “SMART VENT (PRODUCT / MANUFACTURER SERIAL #)

  - X-Ray view of top level & kitchen

  - All beams dropped below ceiling elevation are to be highlighted in RED and dimensioned

  - Cross section of all interior and exterior stair systems with rise and run noted

    - Structure notes if applicable

  - Assume 42” Astria Georgian wood burning fireplace with code compliant venting OR 48” Mendota linear for gas fireplace per client preference

  - Unless noted otherwise, top of windows to align with top of doors:

    - 8’-0” ceiling = 7’-0” door

    - 8’-6” ceiling = 7’-6” door

    - 9’-0”+ ceiling = 8’-0” door

    - Top Floor and / or feature windows and doors to be maximized to the degree possible with MCH direction

  - 1.25” Gypcrete included on all levels above ground level per client direction

  - “Ground Level”, “First level”, “second level”, “third level”

  - Consistent labels for rooms “Bedroom #1”, “Bathroom #1”, “Primary Bedroom”, “Primary Bathroom”

  - Curves and sweeps to include radius and dimensions

  - Include labeled mechanical room and/or storage room on each level

  - Incorporate wish list

  - Final product to include: Permit ready plans to include sealed electronic copy + 3 sealed hard sets with structural included, res check , county plan review waivers signed and sealed, and DWG files.

  - Wish list questions:

    - Soffit and fascia detail

    - Hurricane Shutters?

    - Recessed showers?

    - Recessed screens / Recessed window treatments?

    - Mechanical equipment: HVAC / Generator / pool equipment / solar

    - Exterior rails

    - Gypcrete

    - Flooring

    - Siding

    - Pool / hot tub

- Due diligence to include

  - confirmation of jurisdictional requirements including HOA if applicable

  - Civil – lot boundaries, applicable building restriction lines, flood zone(s), DNREC 4 step process,

  - Small Lot

  - Wetlands

  - Corner lot conditions

  - Topography

  - Dune Elevation

  - Collaboration with the structural engineer

  - Collaboration with the MCH design team